October 16, 2025

Functionality Attestation (FA)

SUBJECT: Cisco Global Certification Team (GCT) Defense Programs (DP) Internal Compliance Review (ICR) Functionality Attestation (FA) of the Cisco Session Border Controller (SBC)

References: 
(a) Defense of Defense Instruction (DoDI) 8100.04, “Department of Defense (DoD) Unified Capabilities (UC),” 09 December 2010
(b) Office of the DoD Chief Information Officer (CIO), “DoD Unified Capabilities Requirements 2013 (UCR 2013) Change 2,” September 2017
(d) Cisco GCT DP, “Cybersecurity Summary Report (CSR) for Cisco Session Border Controller (SBC), Software Rel. IOS XE 17.18”
(e) Test procedures

- Internal Compliance Authority. The Cisco GCT DP is designated as the attestation authority responsible for overseeing and validating the DTR process to ensure adherence to established Cybersecurity (CS) and FA standards.

- Conditions of Attestation. The Cisco SBC running Software Rel. Internetwork

Operating System (IOS) XE 17.18 (System Under Test, or SUT) has been attested for    	joint use and meets the essential High Availability (HA) SBC requirements outlined in    	the UCR 2013, Reference (b). This attestation remains valid under the condition listed

in Table 1 Conditions and will expire if any changes affecting interoperability are made, or on the date specified in the Cisco GCT DP DTR List.

Table 1. Conditions

| TDR # | Description | Operational Impact | Remarks |
| --- | --- | --- | --- |
| CIS-1824-001 | MLPP call fails to produce Audio upon preemption | CLOSED | DISA accepted the Vendor’s POA&M and adjudicated this discrepancy as minor. |

- System Requirements identification and Test Configuration. The SUT, identified 	as the Cisco SBC with Software Rel. IOS XE 17.18 (detailed in Table 2-1), was tested

by Cisco GCT DP. This testing was conducted in functional setup. Figure 2-2 illustrates  	the SUT test configuration, including its components and connectivity schemes, which	were consistently employed for both FA and CS testing.

Table 2-1. System Requirements Identification

| System Identification |  |
| --- | --- |
| System Name | Cisco SBC |
| Increment and/or Version | IOS XE 17.18 |
| Product Category | SBC with High Availability |
| Cisco Point of Contact |  |
| Tracking |  |
| DTR ID | 003 (ISR 4000 Series, ASR 1000 Series, C8000v, and C8000 Series) |
| Requirement Source |  |
| Unified Capabilities Requirement | UCR 2013, Change 2, Sections 2, 4, and 5 |
| Remarks | None |
| Test Organization | Cisco GCT DP |

Figure 2-2. SUT Test Configuration

- System Description

  - General Description. The SUT is an SBC, which performs voice firewall and back-to-back user agent functions. The SBC consists of the voice or video firewall/border controller. The Call Connection Agent (CCA) in the Softswitch (SS) and Local Session Controller (LSC) needs to interact with Assured Services Session Initiation Protocol (AS-SIP) functions in the SBC which:

    - Mediates AS-SIP signaling between a Session Controller (SC) and a Wide Area Network (WAN) SS, and between two SSs.

    - Supports SBC functions, such as Network Address Translation (NAT) and Network Address and Port Translation (NAPT).

    - Supports Internet Protocol (IP) firewall functions.

SBCs can be certified in one of two availability categories: High Availability (HA) and Medium Availability. The SUT is certified for joint use as an HA SBC. The SUT supports IP firewall functions. The Cisco SBC facilitates IP to IP AS-SIP communications between SCs that support the Unified Capabilities Requirements (UCR) Alternative B failover method and their end instruments, when traversing the Unified Capabilities (UC) network. The SBC interfaces with Defense Information Systems Network (DISN) UC systems through an AS-SIP interface and provides IP to IP communications between edge LSC/Enterprise Session Controller (ESC) and core (Softswitch) segments in the UC network. Additional functions include AS-SIP signaling and encryption services to allow end to end communications on the UC network. This SBC solution consists of the Cisco Integrated Services Router (ISR) 4000 series, Aggregation Services Router (ASR) 1000 series, Catalyst (C)8000v, C8200 series, and C8300 series of hardware platforms.

The system consists of the following components:

- SBC

- Combined SBC/Interworking Gateway (IWBC)

- IWG (Interworking Gateway)

The SBC is an intelligent unified communications network border element that provides SBC functions that help enable end-to-end IP-based transport of voice, video, and data between independent unified communications networks and the UC network.

The SBC combined with IWG type deployment provides critical Session Initiation Protocol (SIP) message normalization functionality that allows for interoperability between components within the UC architecture for the Cisco SC. The combined SBC and IWG is an integrated Cisco IOS XE software application that runs on the Cisco ISR 4000 series, ASR 1000 series, C8000v, C8200 series, and C8300 series of hardware platforms.

  - Management Description. The SBC is managed with a site-provided, Security Technical Implementation Guideline (STIG)-compliant, Common Access Card (CAC)-enabled Windows Operating System workstation. After initial configuration, administrative tasks are performed using a Secure Shell (SSH) version (v)2 (SSHv2) connection. A local emergency account is configured for cases when the Active Directory server is unavailable. The emergency administrative account meets password complexity requirements enforced by local policy enforcement.

- Operational Architecture. The DoDIN architecture is structured as a two-level network hierarchy, comprising DISN backbone switches and Service/Agency installation switches. The specific type of switch permitted at any given location is determined by DoD Chief Information Officer and Joint Staff policy, along with subscriber mission requirements. Consequently, the DoDIN architecture incorporates various categories of switches. This notional operational DoDIN architecture, which illustrates where the SUT may be utilized, is depicted in Figure ES.1. UC High Level Operational Framework within the Department of Defense (DoD) Unified Capabilities Master Plan (UC MP).

- Test Details. This attestation is based on functionality testing. Cisco GCT DP conducted functionality testing from 15 May 2025 through 19 May 2025, using test procedures derived from Reference (e). Cisco is tracking on open TDRs found in Table 1. Cisco GCT DP conducted CS testing, with the results documented in a separate report (Reference (d)).

DTR 2 was requested to update the IOS XE software version from 17.12 to 17.15 for the

product component IWBC on the ISR 4461 series of router platforms and update the 
            ESXi software version from 7.0.3 to 8.0.3.

DTR 3 was a request to update the IOS XE Software Release version from 17.15 to 	17.18 for the product components IWBC, IWG, and SBC on the ASR 1006-X, ISR 
 	4461/K9, C8300 series, C8200 series, and 8000v series of router platforms.

- Functionality Status. Table 3 provides the SUT interface functionality status; Table 4 provides the SUT hardware, software and firmware versions; and Table 5
provides the Capability Requirements. 
      
                                       Table 3. SUT Interface Status

| Network Management Interfaces |  |  |  |
| --- | --- | --- | --- |
| Interface | Applicability | Status | Remarks |
| IEEE 802.3i (10BaseT UTP) | C | Met | The SUT met the critical CRs and FRs for the IEEE 802.3i. |
| IEEE 802.3u (100BaseT UTP) | C | Met | The SUT met the critical CRs and FRs for the IEEE 802.3u. |
| Network Interfaces |  |  |  |
| Interface | Applicability | Status | Remarks |
| IEEE 802.3ab (1000BaseT UTP) | C | Met | The SUT met the critical CRs and FRs for the IEEE 802.3ab. |
| IEEE 802.3z (1000BaseX Fiber) | C | Met | The SUT does not support this interface. |
| IEEE 802.3ae (10GBaseX) | C | Met (See Note 2) | The SUT met the critical CRs and FRs for the IEEE 802.3ae. |
| IEEE 802.3an (10GBaseT UTP) | C | Met | The SUT met the critical CRs and FRs for the IEEE 802.3an. |
| IEEE 802.3ba (100GBaseX Fiber) | C | Not Tested | The SUT does not support this interface. |
| NOTE(S): - Per the UCR, the SUT can support any one of these conditional interfaces. Table 3 depicts the SUT high-level requirements category and the applicability of each sub-requirement. - The 10GBaseX WAN interfaces were not tested but are included and certified for joint use on only the 4000 Series and 8300 Series. IWBCs based on Cisco GCT DP’s ICR analysis of CER test data |  |  |  |

Table 4. SUT Hardware/Software/Firmware Version Identification

| Product Identification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Product Name | Cisco SBC |  |  |  |
| Software Release | IOS XE 17.18 |  |  |  |
| UCR Product Type(s) | SBC |  |  |  |
| Product Description | The SBC is an intelligent unified communications network border element that provides SC functions that help enable end-to-end IP-based transport of voice, video, and data between independent unified communications networks and the DoDIN network. The Cisco SBC combined with IWG type deployment provides critical SIP message normalization functionality that allows for interoperability defined in the UCR (CORE) change 2. The combined SBC and IWG is an integrated Cisco IOS XE software application that runs on the Cisco ISR 4000 series, ASR 1000 series, C8000v, C8200 series, and C8300 series of hardware platforms. |  |  |  |
| Product Components | Component Name (See notes 1 and 2.) | Sub-Component | Tested Version | Function |
| IWBC | 4321 ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWBC | 4331 ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |

| Product Components | Component Name (See notes 1 and 2.) | Sub-Component | Tested Version | Function |
| --- | --- | --- | --- | --- |
| IWBC | 4351 ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWBC | 4431 ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWBC | 4451-X ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWBC | 4461 ISR | NA | IOS XE 17.18.1a | Provides connectivity to AS-SIP trunks. (See note 3) |
| IWBC | ASR 1001-X | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWBC | ASR 1002-X | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWBC | ASR 1004 | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWBC | ASR 1006 | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWBC | ASR1006-X | NA | IOS XE 17.18.1a | Provides connectivity to AS-SIP trunks. (See note 4) |
| IWBC | C8200-1N-4T C8200L-1N-4T | NA | IOS XE 17.18.1a | Provides connectivity to AS-SIP trunks. (See note 4) |
| IWBC | C8300-1N1S-4T2X | NA | IOS XE 17.18.1a | Provides connectivity to AS-SIP trunks. (See note 4) |
| IWBC | C8300-2N2S-4T2X | NA | IOS XE 17.18.1a | Provides connectivity to AS-SIP trunks. (See note 4) |
| IWBC | C8300-1N1S-6T | NA | IOS XE 17.18.1a | Provides connectivity to AS-SIP trunks. (See note 4) |
| IWBC | C8300-2N2S-6T | NA | IOS XE 17.18.1a | Provides connectivity to AS-SIP trunks. (See note 4) |
| IWBC | C8000v | ESXi Server Host | IOS XE 17.18.1a | Provides connectivity to AS-SIP trunks. (See note 4) |
| SBC | 4321 ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |

| Product Components | Component Name (See notes 1 and 2.) | Sub-Component | Tested Version | Function |
| --- | --- | --- | --- | --- |
| SBC | 4331 ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| SBC | 4351 ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| SBC | 4431 ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| SBC | 4451-X ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| SBC | 4461 ISR | NA | IOS XE 17.18.1a | Provides connectivity to AS-SIP trunks. (See note 3) |
| SBC | ASR 1001-X | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| SBC | ASR 1002-X | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| SBC | ASR 1004 | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| SBC | ASR 1006 | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| SBC | ASR1006-X | NA | IOS XE 17.18.1a | Provides connectivity to AS-SIP trunks. (See note 4) |
| IWG | 4321 ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWG | 4331 ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWG | 4351 ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWG | 4431 ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWG | 4451-X ISR | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWG | 4461 ISR | NA | IOS XE 17.18.1a | Provides connectivity to AS-SIP trunks. (See note 3) |
| IWG | ASR 1001-X | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |

| Product Components | Component Name (See notes 1 and 2.) | Sub-Component | Tested Version or Release(?) | Function |
| --- | --- | --- | --- | --- |
| IWG | ASR 1002-X | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWG | ASR 1004 | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWG | ASR 1006 | NA | IOS XE 17.12.2 | Provides connectivity to AS-SIP trunks. |
| IWG | ASR1006-X | NA | IOS XE 17.18.1a | Provides connectivity to AS-SIP trunks. (See note 4) |
| IWBC | C8000v | ESXi Server Host | IOS XE 17.18.1a | Provides connectivity to AS-SIP trunks. (See note 4) |
| IWBC, IWG, SBC | ESXi Server Host | NA | ESXi 8.0.3, 2402251 |  |
| IWBC, IWG, SBC | Management Workstation (Site-provided) | NA | Windows 11 Enterprise, v22H2 |  |
| IWBC, IWG, SBC | Management Workstation (Site-provided) | NA | ActivClient 7.2.1.46 |  |
| IWBC, IWG, SBC | Management Workstation (Site-provided) | NA | Axway Desktop Validator SE 5.0.3.0052 |  |
| IWBC, IWG, SBC | Management Workstation (Site-provided) | NA | SecureCRT v9.3.2 (x64 build 2978) |  |
| NOTE(S): - The SUT was initially certified with Software Release IOS XE 17.12. The IOS XE Software Release version was updated via subsequent DTR(s) as follows: DTR002 from 17.12 to 17.15; DTR003 from 17.15 to 17.18. - Cisco GCT DP tested the bolded and underlined components/sub-components. The remaining items in the family series were not directly tested but are certified for joint use because they share the same software and similar hardware as the tested components. Based on Cisco GCT DP's analysis, they are functionally identical for IO certification purposes. - For the ISR 4461 series, the IOS XE Software Release was updated from 17.12 to 17.15 with DTR002, and from 17.15 to 17.18 with DTR0003, both via analysis (no testing). These updates were supported by prior testing and certification of the respective software versions during the initial Cisco Softswitch (SS) test event under DTR00X. - For the ASR 1006-X, C8300, C8200, and C8000v router platforms, the IOS XE Software Release was updated from 17.12 to 17.15 with DTR001, and from 17.15 to 17.18 with DTR003. The initial update was based on testing and certification during the DTR00X Cisco ESC test event, while the subsequent update to 17.18 was approved via analysis (no additional testing), supported by previous DTR00X certification. - With DTR002, the ESXi Software Release version was updated from 7.0.3 to 8.0.3 via analysis (no testing) based on testing and certification of this software version during the initial test event for Cisco SS. |  |  |  |  |

Table 5. SUT Capability Requirements and Functional Requirements Status

| CR/FR ID | UCR Requirement (High-Level) | Applicability (See note 1.) | UCR 2013 Change 2 | Status |
| --- | --- | --- | --- | --- |
| 1 | SC and SS Failover and Recovery | Required | 2.6 | Met (See note 2.) |
|  | SC Failover: Alternative A: The SC-Generated OPTIONS Method | Optional | 2.6.1 | Not Tested (See note 2.) |
|  | SC Failover: Alternative B: The SBC-Generated OPTIONS Method | Required | 2.6.2 | Met (See note 2.) |
| 2 | Product Interface | Required | 2.7 | Met |
|  | Internal Interfaces | Required | 2.7.1 | Met |
|  | External Interfaces | Required | 2.7.2 | Met |
|  | EMS Interfaces | Required | 2.7.4 | Met |
| 3 | AS-SIP IP Gateway | Conditional | 2.11 | Met |
|  | AS-SIP IP Gateway Media Interworking | Conditional | 2.11.2 | Met |
| 4 | Session Border Controller (SBC) Enterprise Session Controller (ESC) | Required | 2.12.4 | Met (See note 4.) |
|  | General SBC Functionality | Required | 2.12.4.1 | Met (See note 3.) |
|  | Enclave-Fronting SBC Functionality | Required | 2.12.4.2 | Met (See note 3.) |
|  | Enclave-Fronting SBC Functionality within the core infrastructure | Required | 2.12.4.3 | Met (See note 3.) |
| 5 | Remote Media Gateway | Conditional | 2.16 | Met |
|  | Remote MG | Conditional | 2.16.13 | Met |
| 6 | Session Border Controller | Required | 2.17 | Partially Met (See note 4.) |
|  | AS-SIP Back-to-Back User Agent | Required | 2.17.1 | Met |
|  | Call Processing Load | Required | 2.17.2 | Met |
|  | Network Management | Required | 2.17.3 | Met |
|  | DSCP Policing | Optional | 2.17.4 | Not Tested (See note 5.) |
|  | Codec Bandwidth Policing | Optional | 2.17.5 | Met |
|  | Availability | Required | 2.17.6 | Partially Met (See note 4.) |
|  | IEEE 802.1Q Support | Required | 2.17.7 | Met |
|  | Packet Transit Time | Required | 2.17.8 | Met |
|  | H.323 Support | Conditional | 2.17.9 | Met |
|  | SBC Requirements to Support Remote MG | Conditional | 2.17.10 | Met |
|  | SBC Support for Multiple SCs | Optional | 2.17.11 | Met |
| 7 | Network Management | Required | 2.19 | Met (See note 7.) |

| CR/FR ID | UCR Requirement (High-Level) | Applicability (See note 1.) | UCR 2013 Change 2 | Status |
| --- | --- | --- | --- | --- |
| 7 | General Management | Required | 2.19.1 | Met |
|  | Requirements for FCAPS Management | Required | 2.19.2 | Met (See note 4.) |
| 8 | Cybersecurity | Required | 4.2 | Met (See note 7.) |
| 9 | Internet Protocol version 6 | Required | Table 5.2-1 | Met |
| 10 | AS-SIP 2013 | Required | AS-SIP | Met |
| NOTE(S): - The annotation of ‘required’ refers to a high-level requirement and applicability of each sub-requirement. Refer to the UCR 2013, Change 2 (b). - The UCR states that each SC and SBC pairing shall support OPTIONS-based failover by at least one of the following methods. 1. Alternative A: The SC-Generated OPTIONS Method or 2. Alternative B: The SBC-Generated OPTIONS Method. The SUT supports Alternative B SBC-Generated OPTIONS Method for failover. - This requirement applies specifically to an SBC fronting an ESC. - The SUT met the requirements for High and Medium Availability with testing and the Vendor’s LoC, with the following exception, documented in Table 1: MLPP calls fail to produce audio upon preemption. - The SUT does not support the optional DSCP Policing requirement. - The SUT met the requirements for High and Medium Availability with testing and the (GCT DP) Vendor’s LoC, with the following exception, documented in Table 2: <POA&M Number> A GCT DP test conducted CS testing and published the results in a separate report, Reference (c). In addition to the CS testing conducted by the test team, the SUT met the requirements in section 4 with the (GCT DP) Vendor’s LoC. |  |  |  |  |

- Point of Contact (POC): Cisco Global Certification Team, Defense Programs

7025-2 Kit Creek Road, Research Triangle Park, North Carolina 27709

Cisco GCT DP Email: gct-dp-icr@cisco.com

Appendix 1. Acronym List

| Acronym List |  |
| --- | --- |
| ASR | Aggregation Services Router |
| AS-SIP | Assured Services Session Initiation Protocol |
| APL | Approved Products List |
| BaseT | Megabit (Baseband Operation, Twisted Pair) Ethernet |
| BaseX | Megabit Ethernet over Fiber or Copper |
| C | Catalyst |
| CAC | Common Access Card |
| CAR | Cybersecurity Assessment Report |
| CCA | Call Connection Agent |
| CER | Customer Edge Router |
| CR | Capability Requirement |
| CIO | Chief Information Officer |
| CS | Cybersecurity |
| DISN | Defense Information Systems Network |
| DoD | Department of Defense |
| DODI | Department of Defense Instruction |
| DODIN | Department of Defense Information Network |
| DP | Defense Programs |
| DTR | Desktop Review |
| ESC | Enterprise Session Controller |
| ESXi | Elastic Sky X integrated |
| FR | Functional Requirement |
| GBaseT | Gigabit Ethernet over Copper |
| GBaseX | Gigabit Ethernet over Fiber or Copper |
| GCT | Global Certification Team |
| HA | High Availability |
| ICR | Internal Compliance Review |
| IEEE | Institute of Electrical and Electronic Engineer |
| ID | Identification |
| IOS XE | Internetwork Operating System XE |
| IP | Internet Protocol |
| ISR | Integrated Service Router |
| IWBC | Interworking Border Controller |
| IWG | Interworking Gateway |
| LoC | Letter of Compliance |
| LSC | Local Session Controller |
| MLPP | Multilevel Precedence and Preemption |
| NAT | Network Address Translation |
| NAPT | Network Address and Port Translation |
| R | Required |
| Rel. | Release |
| SBC | Session Border Controller |
| SC | Session Controller |
| SIP | Session Initiation Protocol |
| SS | Softswitch |
| STIG | Security Technical Implementation Guideline |
| SUT | System Under Test |
| UC | Unified Capabilities |
| UC MP | Unified Capabilities Master Plan |
| UCR | Unified Capabilities Requirements |
| UTP | Unshielded Twisted Pair |
| WAN | Wide Area Network |