Cisco Global Certification Team (GCT) Defense Programs (DP) Internal Compliance Review (ICR) Military Unique Deployment Guide (MUDG) for Session Border Controller (SBC)

| Version | Date | Change | Editor |
| --- | --- | --- | --- |
| 1.0 | February 2026 | Initial Version | GCT DP Collaboration |

Cisco Global Certification Team, Defense Programs

7025-2 Kit Creek Road, Research Triangle Park, North Carolina 27709

Cisco GCT DP Email: gct-dp-icr@cisco.com

This document provides comprehensive guidance for both Cisco engineers and customers on the deployment of the Cisco SBC certified solution. It is intended to supplement official product documentation by detailing specific configuration parameters and implementation guidelines essential for achieving SBC certification.

The procedures and examples outlined in this guide illustrate recommended configuration practices. For optimal results and compliance, all components of the Cisco SBC solution should be deployed in accordance with Cisco best practices, in conjunction with the requirements specified herein.

Table of Contents

Configuration Guidelines for Cisco SBC	3

Conditions of Fielding	3

Hardening and Cybersecurity Configuration	4

Configuration Checklist	8

Redundancy and Media Failover	9

PKI Certificates	14

Call Routing Configuration	19

SBC Configuration Example	24

Related Documentation	30

List of Acronyms	31

Figure 1: SUT Test Configuration

# Configuration Guidelines for Cisco SBC

Prior to utilizing this guide, it is strongly recommended to acquire and review all relevant administrative and best practice documentation for each product referenced herein. Ensuring familiarity with these resources will help facilitate adherence to cisco best practices, as well as any required ancillary products that are integral to this solution.

The configuration examples provided in this guide are tailored to the Unified Capabilities Requirements (UCR) 2013, Change 2 test environment and are intended solely for illustrative purposes. Actual configuration parameters and command syntax may differ depending on the specific platform and deployment scenario. Users should consult platform-specific documentation to validate and adapt commands as necessary.

# Conditions of Fielding

Upon deployment into an operational environment, security controls must be established and continuously maintained in accordance with the Cisco GCT DP ICR Cybersecurity Assessment Report (CAR). This is essential to ensure that the system meets the risk management standards required by the site's Designated Approving Authority.

# Hardening and Cybersecurity Configuration

The following sections outline the procedures necessary to secure the Cisco SBC in accordance with Cybersecurity requirements, referencing the product-specific Security Technical Implementation Guide (STIG).

SBC Integration with Terminal Access Controller Access Control System Plus (TACACS+)

Server.

Below is an example configuration for integrating the SBC with a TACACS+ server, which facilitates centralized authentication, authorization, and accounting.

TACACS+ Server Configuration Example:

tacacs-server <Name>

address ipv4 <ISE ESR IP Address>

tacacs-server key 7 <password hash>

syslog server config

logging <Syslog Server IP Address>

Auntentication, Authorization, Accounting (AAA) Configuration Example:

aaa new-model

aaa group server tacacs+ <ISE_GROUP>

server name <TACACS+ server name>

aaa authentication login AAA group <ISE_GROUP> local

aaa authentication enable default group <ISE_GROUP> enable

aaa authorization config-commands

aaa authorization exec AAA group <ISE_GROUP> local

aaa authorization commands 0 AAA group <ISE_GROUP> local

aaa authorization commands 1 AAA group <ISE_GROUP> local

aaa authorization commands 15 AAA group <ISE_GROUP> local

line vty 0 4

authorization commands 0 AAA

authorization commands 1 AAA

authorization commands 15 AAA

authorization exec AAA

login authentication AAA

The SBC can be deployed using SecureCRT to enforce strong multi-factor authentication (MFA). For detailed MFA configuration guidance, refer to the accompanying documents titled “SBC23_MFA_Config.txt” and “SBC23_ISE_IPSEC.txt” and the following configuration examples.

SBC MFA Configuration Example

aaa new-model

aaa session-id common

tacacs server <ISE3>

address ipv4 <105.16.57.144>

key 7 <101A585D44343B381816232A202F3021>

aaa group server tacacs+ <ISE_GROUP>

server name <ISE3>

aaa authentication login AAA group <ISE_GROUP> local

aaa authentication enable default group <ISE_GROUP> enable

aaa authorization config-commands

aaa authorization exec AAA group <ISE_GROUP> local

aaa authorization commands 0 AAA group <ISE_GROUP local>

aaa authorization commands 1 AAA group <ISE_GROUP> local

aaa authorization commands 15 AAA group <ISE_GROUP> local

crypto pki trustpoint <DODJITCCA63>

enrollment terminal

revocation-check ocsp

ocsp disable-nonce

authorization username alt-subjectname userprinciplename

hash sha1

crypto pki trustpoint <DODJITCCA65>

enrollment terminal

revocation-check none

authorization username alt-subjectname userprinciplename

hash sha256

ip ssh server certificate profile

server

trustpoint sign <DODJITCCA60>

user

trustpoint verify <DODJITCCA63>

trustpoint verify <DODJITCCA65>

trustpoint verify <DODJITCROOTCA3>

ip ssh server algorithm authentication publickey password

Ip ssh server algorithm pubic key x509v3-rsa2048-sha256

<configuration continued>

line vty 0 4

exec-timeout 30 0

privilege level 15

password 7 <xxxxxxxxxxxxxxxxxxxxxxxxxxxxxx>

no activation-character

authorization commands 0 AAA

authorization commands 1 AAA

authorization commands 15 AAA

authorization exec AAA

logging synchronous

login authentication AAA

transport preferred none

transport input ssh

transport output ssh

SBC ISE IPSEC Configuration Example

ip tacacs source-interface GigabitEthernet2

crypto isakmp policy <10>

encryption aes

hash sha256

group <14>

crypto isakmp identity dn

crypto isakmp keepalive 10

crypto isakmp aggressive-mode disable

crypto isakmp profile <ISAKMP-ISE>

self-identity address

ca trust-point <DODJITCROOTCA67>

match identity address <105.16.57.144 255.255.255.255>

local-address <105.16.65.52>

crypto ipsec transform-set ISE esp-aes esp-sha256-hmac

mode transport

crypto ipsec df-bit clear

no crypto ipsec nat-transparency udp-encapsulation

<configuration continued>

crypto map ISEmap local-address GigabitEthernet2

crypto map ISEmap 10 ipsec-isakmp

set peer <105.16.57.144>

set security-association lifetime seconds 28800

set transform-set <ISE>

match address <110>

ip access-list extended <110>

20 permit ip host <105.16.65.52> host <105.16.57.144>

interface GigabitEthernet2

crypto map <ISEmap>

SBC Integration with SYSLOG Server

To ensure robust audit logging, integrate the SBC with a SYSLOG server as shown below.

SYSLOG Server Configuration Example:

logging source-interface Loopback0

logging host tls-profile <SYSLOG_TLS>

tls-version TLSv1.2

client-id-trustpoint Syslog

logging host <2.2.2.2> transport tls profile SYSLOG_TLS

Two trustpoints must be configured using the Intermediate and Root CA certificates of the SYSLOG server to establish a trusted TLS connection.

SBC Session Timeout and Access Configuration

To ensure compliance with security and operational requirements, configure session timeouts and access controls on the SBC as demonstrated in the example below.

Example: SBC IP Session Timeouts and Access Configuration

ip tcp synwait-time 10

no ip ftp

no ip tftp

ip ssh time-out 60

ip ssh source-interface <interface>

ip ssh logging events

ip ssh version 2

no ip http server

no ip https server

SBC Maintenance and Management Port Configuration

Properly configuring maintenance and management ports is essential for secure administrative access. The following example illustrates best practices for console, auxiliary, and virtual terminal (VTY) lines.

Example: SBC Maintenance and Management Ports Configuration

line con 0

password 7 <XXXXXXXXXXXXXXXXXXX>

activation-character 13

transport output ssh

stopbits 1

line vty 0 4

session-timeout 10

exec-timeout 0 0

password 7 <XXXXXXXXXXXXXXXXXXXX>

no activation-character

authorization commands 0 AAA

authorization commands 1 AAA

authorization commands 15 AAA

authorization exec AAA

logging synchronous

login authentication AAA

transport preferred none

transport input ssh

transport output telnet ssh

stopbits 1

# Configuration Checklist

The following checklist outlines the essential steps required to establish a fully operational SBC. Adherence to these steps ensures compliance with security, redundancy, and operational best practices.

1. Redundancy and Media Failover Support

(Applicable when deploying a highly available SBC solution)

- Enable inter-device redundancy to facilitate high availability.

- Configure Virtual Router Redundancy Protocol (VRRP) for automatic failover.

- Enable voice redundancy mechanisms to ensure uninterrupted media flow.

- Save the running configuration to persistent storage.

- Perform a system reload to apply changes.

2. Cryptographic Configuration

- Set the system hostname and configure the IP domain name.

- Generate an RSA key pair with a 2048-bit modulus for SIP trunk security.

- Configure a PKI trustpoint labeled sip-trunk:

- Specify the subject name.

- Attach the generated RSA key pair.

- Disable FQDN, IP address, and serial number fields as appropriate.

- Generate a certificate signing request (CSR) for the sip-trunk trustpoint.

- Configure a PKI trustpoint for the root CA (sip-trunk-root):

- Import and authenticate the root CA certificate.

- Authenticate and import the signing CA certificate into the sip-trunk trustpoint.

- Import the SBC’s signed certificate into the sip-trunk trustpoint.

- Configure the SIP-UA to utilize the sip-trunk trustpoint for TLS signaling.

3. Call Routing Configuration

- Disable IOS toll fraud prevention features as required by the deployment scenario.

- Enable border-element mode for SIP trunking functionality.

- Save the configuration.

- Reload the system to activate changes.

- Configure the voice service voip and sip-ua commands for call processing.

- Define SIP URI classes (minimum of two required):

  - cca0

  - sbc0

- Configure SIP profiles for advanced SIP feature manipulation.

- Configure dial-peers for call routing.

- Enable media inactivity detection to monitor and manage call media streams.

# Redundancy and Media Failover

- Cisco SBC redundancy is achieved by leveraging the Cisco Unified Border Element (CUBE) High Availability (HA) feature. High availability is implemented using a First Hop Redundancy Protocol (FHRP), such as VRRP, in conjunction with an inter-device communication protocol.

- VRRP establishes an Active/Standby relationship between two SBC routers, allowing both devices to share a common virtual IP address. The Active and Standby routers continuously exchange status messages to monitor system health and maintain synchronization. Critical SIP session state information is checkpointed in real time from the Active to the Standby router via the inter-device communication protocol. This enables the Standby router to assume full call processing responsibilities with minimal service interruption in the event of a planned or unplanned outage of the Active router.

Below are sample configuration snippets for deploying VRRP version 3 on IOS XE-based CUBE routers.

IPsec Tunnel Configuration for Cisco SBC Redundancy

When establishing redundancy between Cisco SBC HA pairs, it is essential to configure an IPsec tunnel to ensure that all inter-device communications are securely encrypted. The IPsec tunnel safeguards the synchronization of SIP session data and status messages exchanged between the Active and Standby routers, thereby maintaining confidentiality and integrity in compliance with government security standards.

For detailed configuration guidance, refer to the accompanying document titled "Redundancy_IPSEC_Config.txt" and the following configuration examples.

crypto ikev2 proposal aes-cbc-256-proposal

encryption aes-cbc-256

integrity sha256

group <14>

crypto ikev2 policy <REDUNDANCY_Policy>

match address local <10.10.10.6>

proposal aes-cbc-256-proposal

crypto ikev2 profile <REDUNDANCY_Profile>

description <IKEv2 profile for redundant IPsec tunnel>

match address local <10.10.10.6>

match identity remote address <10.10.10.5 255.255.255.255>

authentication remote rsa-sig

authentication local rsa-sig

pki trustpoint <DODJITCCA60>

crypto isakmp aggressive-mode disable

crypto ipsec transform-set REDUNDANCY_Transform esp-aes 256 esp-sha256-hmac

mode transport

crypto map <REDUNDANCY_MAP 1> ipsec-isakmp

set peer <10.10.10.5>

set transform-set <REDUNDANCY_Transform>

set pfs group <14>

set ikev2-profile <REDUNDANCY_Profile>

match address <101>

interface GigabitEthernet0/0/2

crypto map <REDUNDANCY_MAP>

access-list 101 permit ip <10.10.10.6 0.0.0.0 10.10.10.5 0.0.0.0>

Sample Configuration for CUBE Redundancy on IOS XE Routers

This section provides a sample configuration for implementing CUBE HA using VRRP on IOS XE-based routers. For further reference, consult the official Cisco documentation: Cisco Unified Border Element High Availability (HA) Using VRRP – Configuration Example

Step 1: Enable Inter-Device Redundancy

Inter-device redundancy establishes the communication and state synchronization link between the Active and Standby routers. This configuration ensures that the Standby router can immediately take over call processing in the event the Active router becomes unavailable.

Implement this configuration on both the Active and Standby SBC routers.

redundancy inter-device

scheme standby <EBC-RED>

redundancy

mode none

application redundancy

group 1

name sbc-ha

priority 100 failover threshold 75

timers delay 60 reload 90

control GigabitEthernet0/0/0 protocol 1

data GigabitEthernet0/0/0

track 1 shutdown

track 2 shutdown

cdp run

track 1 interface GigabitEthernet0/0/1 line-protocol

track 2 interface GigabitEthernet0/0/2 line-protocol

ip tcp synwait-time 10

Step 2: Enable VRRP and Configure Redundant Interfaces

Configure VRRP version 3 and apply redundancy settings to the relevant interfaces on both the Active and Standby routers. The following example details interface configurations for each router.

Active Router Configuration

interface GigabitEthernet0/0/0

description data and control

ip address <10.10.10.1 255.255.255.252>

negotiation auto

ipv6 address <2110:10:10::1/125>

fhrp delay minimum 5

fhrp delay reload 25

interface GigabitEthernet0/0/1

ip address <201.2.1.67 255.255.255.240>

no ip redirects

no ip unreachables

no ip proxy-arp

ip route-cache same-interface

ip ospf authentication message-digest

negotiation auto

no ipv6 redirects

no ipv6 unreachables

cdp enable

redundancy rii <20>

redundancy group 1 ip <201.2.1.66> exclusive

interface GigabitEthernet0/0/2

description inside interface

ip address <105.16.97.52 255.255.255.0>

no ip redirects

no ip unreachables

no ip proxy-arp

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 7 <XXXXXXXXXXXXXXX>

ip ospf 132 area 132

negotiation auto

cdp enable

redundancy rii <10>

redundancy group 1 ip <105.16.97.51> exclusive

sbc redundancy-group 1 tcp

Standby Router Configuration

interface GigabitEthernet0/0/0

description data and control

ip address <10.10.10.2 255.255.255.252>

negotiation auto

ipv6 address <2110:10:10::2/125>

fhrp delay minimum 5

fhrp delay reload 25

interface GigabitEthernet0/0/1

description outside interface

ip address <201.2.1.68 255.255.255.240>

no ip redirects

no ip unreachables

no ip proxy-arp

ip route-cache same-interface

ip ospf authentication message-digest

<configuration continued>

negotiation auto

no ipv6 redirects

no ipv6 unreachables

cdp enable

redundancy rii <20>

redundancy group 1 ip <201.2.1.66> exclusive

interface GigabitEthernet0/0/2

description inside interface

ip address <105.16.97.53 255.255.255.0>

no ip redirects

no ip unreachables

no ip proxy-arp

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 7 <XXXXXXXXXXXXXX>

ip ospf 132 area 132

negotiation auto

cdp enable

redundancy rii <10>

redundancy group 1 ip <105.16.97.51> exclusive

sbc redundancy-group 1 tcp

To configure HA failover, you will need to add VRRP settings to the data and control interfaces, and ensure voice redundancy is enabled.

1. Interface Configuration for HA Failover

Add the following VRRP configuration to the GigabitEthernet0/0/2 interface. This example assumes this is for the primary SBC, with a higher priority.

interface GigabitEthernet0/0/2

description data and control

ip address <10.10.10.1 255.255.255.252>

media-type rj45

negotiation auto

ipv6 address <2110:10:10::1/125>

fhrp delay minimum 5

fhrp swlay reload 25

For the secondary SBC, configure the same VRRP group number (e.g., vrrp 10) but with a lower priority (e.g., priority 90).

2. Voice Redundancy

To ensure media state information is shared between the SBCs in the pair, apply the following commands:

voice service voip

redundancy-group 1

3. Save Configuration and Reload

After applying the configurations to both SBCs, it is essential to save the changes and reload both devices for the redundancy configuration to become active.

copy running-config startup-config

reload

# PKI Certificates

Any pre-existing cryptographic configurations should be disregarded to prevent conflicts and ensure a clean implementation of the specified PKI architecture.

Step 1: Configure Hostname and Domain Name

The initial step involves configuring a unique hostname and a fully qualified domain name (FQDN) for each Session Border Controller (SBC). This configuration is critical for proper identification within the network and for the generation of certificate signing requests (CSRs).

hostname <SBC-Example>

ip domain-name <example.com>

It is imperative to configure a hostname and domain name appropriate to your specific operational environment. The combination provided in the example (SBC-Example and example.com) would result in a combined FQDN of SBC-Example.example.com.

Step 2: Generate Required RSA Keys

This step details the generation of a Rivest-Shamir-Adleman (RSA) key pair, which is fundamental for cryptographic operations, including the signing of certificate requests.

SBC-Example(config)#crypto key generate rsa modulus 2048 label sip-trunk-key

The modulus 2048 parameter specifies a key length of 2048 bits, which is a mandatory requirement to comply with UCR 2013, Change 2, and other applicable security standards.

To enable the backup of cryptographic keys to a secure, external location, the exportable keyword must be appended to the crypto key generate command. For example: crypto key generate rsa modulus 2048 label sip-trunk-key exportable.

Step 3: Configure a Trustpoint for CSR Generation

Trustpoints serve as the designated configuration areas within the Cisco IOS environment for storing and managing certificate data. The establishment of a trustpoint is the prerequisite for initiating the certificate enrollment process.

crypto pki trustpoint sip-trunk-cert

enrollment terminal

serial-number none

fqdn none

ip-address none

subject-name CN=SBC-Example.<example.com,O=CISCO,OU=GGSG,L=RTP,S=NC,C=US,O=US>

revocation-check none

rsakeypair sip-trunk-key

Step 4: Generate a Certificate Signing Request (CSR)

Once the trustpoint has been accurately configured, the subsequent action is to generate a CSR. This CSR is a digitally signed request that will be submitted to a CA for the issuance of a digital certificate specific to this SBC.

SBC-Example(config)#crypto pki enroll sip-trunk-cert

% Start certificate enrollment ..

% The subject name in the certificate will include: SBC-Example.example.com,O=CISCO,OU=GGSG,L=RTP,S=NC,C=US

% The fully-qualified domain name will not be included in the certificate

Display Certificate Request to terminal? [yes/no]: yes

Certificate Request follows:

ZGFtLWN1YmUuY2lzY28uY29tMIIBIjANBgkqhkiG9w0BAQEFAAOCAQ8AMIIBCgKC /T7QNh04hpnVQpR21F0Q2v1I7+YR2M75H4iG5pSL6RrFDtCFnRXNFytZcrr6ZCFDZaswM7kwquSt+sN1EzvAAKpklTo3ZPtP0Jfy96jVAaqCaSHRiaErL5HvGh5Pwp9cWINsvehVfuXFKxWuUrN8vExfrpak1z8P+bd9ws1ZaXwKi9EIbeqHsjOVDK3XICqOsq4m8W/DgO9KX9cv6Su8vq/nOMyDwmQPzr2NTrppOvJja7fGWNeZHQo=

The output displayed above has been truncated for conciseness and readability.

Upon the display of the certificate request in the terminal, it is imperative to copy and paste the entire output into a text editor. Cisco IOS does not automatically include the standard "BEGIN CERTIFICATE REQUEST" and "END CERTIFICATE REQUEST" header and footer lines. It is strongly recommended that these headers be manually added when saving the file to ensure proper formatting for the Certificate Authority. For example:

-----BEGIN CERTIFICATE REQUEST-----

EwNydHAxDjAMBgNVBAoTBWNpc2NvMQ0wCwYDVQQLEwRnZ3NnMRwwGgYDVQQDExNhZGFtLWN1YmUuY2lzY28uY29tMIIBIjANBgkqhkiG9w0BAQEFAAOCAQ8AMIIBCgKC+bd9ws1ZaXwKi9EIbeqHsjOVDK3XICqOsq4m8W/DgO9KX9cv6Su8vq/nOMyDwmQPzr2NTrppOvJja7fGWNeZHQo=

-----END CERTIFICATE REQUEST-----

The output displayed here has been truncated for readability.

Step 5: Authenticate CA Certificates

Before the SBC's own certificate can be imported into the previously created trustpoint, it is necessary to authenticate the certificates of the CAs that form the trust chain. This is typically a multi-step process involving multiple CA servers. To proceed, the following information is required:

Names of CA Servers: Identify the names of the CA servers responsible for issuing or "signing" the SBC's certificate signing request. Common examples may include "DODRootCA2" and "DODCA27," although these names are subject to frequent change. There will almost always be more than one server in a complete trust chain.

CA Certificates (PEM Format): Obtain the certificates from the aforementioned CA servers. These certificates must be in Privacy-Enhanced Mail (PEM) format, which is consistent with the format of the certificate signing request shown in previous steps. The Certificate Authority should be able to provide these certificates.

Procedure:

Import the Root CA Certificate: Begin by importing the certificate of the Root CA server, which represents the highest level in the certificate trust chain. To accomplish this, a new trustpoint must be created specifically for the Root CA.

crypto pki trustpoint sip-trunk-root

enrollment terminal

revocation-check none

Next, authenticate the Root CA by pasting its PEM-formatted certificate data into the terminal.

SBC-Example(config)# crypto pki authenticate sip-trunk-root

Enter the base 64 encoded CA certificate.

End with a blank line or the word "quit" on a line by itself

-----BEGIN CERTIFICATE-----

LkNsYXNzIDMgUHVibGljIFByaW1hcnkgQ2VydGlmaWNhdGlvbiBBdXRob3JpdHkwHhcNOTcwNDE3MDAwMDAwWhcNMTExMDI0MjM5NTkZCBujEfMB0GA1UEChMWVmVy…1RPuKSvD5HKNRO3RrCAJLeH24RkFOLA9D59/+J4C3IYChmFOJl9en5IeDCSk9dBwE88mw0M9SR2egi5SX7w+xmYpAY5Okiy8RnUDgqxz6dl+C2fvVFIa

-----END CERTIFICATE-----

quit

The output displayed here has been truncated for readability.

Upon successful pasting, the system will display certificate attributes and prompt for acceptance:

Certificate has the following attributes:

Fingerprint MD5: EBA2163C A68A9B7C 3D301CFB FEAC1BA1

Fingerprint SHA1: AFDF3A6A 9937EC74 CEF36D97 F9106626 162F15E7

% Do you accept this certificate? [yes/no]:yes

Trustpoint CA certificate accepted.

% Certificate successfully imported

Import the Signing CA Certificate: Subsequently, import the certificate of the signing CA. This certificate will be imported into the original trustpoint that was utilized to generate the CSR. This is achieved by copying and pasting the PEM-formatted certificate data into the terminal.

SBC-Example(config)#crypto pki authenticate sip-trunk-cert

Enter the base 64 encoded CA certificate.

End with a blank line or the word "quit" on a line by itself

-----BEGIN CERTIFICATE-----

LkNsYXNzIDMgUHVibGljIFByaW1hcnkgQ2VydGlmaWNhdGlvbiBBdXRob3JpdHkwHhcNOTcwNDE3MDAwMDAwWhcNMTExMDI0MjM5NTkZCBujEfMB0GA1UEChMWVmVy1RPuKSvD5HKNRO3RrCAJLeH24RkFOLA9D59/+J4C3IYChmFOJl9en5IeDCSk9dBwE88mw0M9SR2egi5SX7w+xmYpAY5Okiy8RnUDgqxz6dl+C2fvVFIa

-----END CERTIFICATE-----

quit

The output displayed here has been truncated for readability.

The system will again display certificate attributes:

Certificate has the following attributes:

Fingerprint MD5: EBA2163C A68A9B7C 3D301CFB FEAC1BA1

Fingerprint SHA1: AFDF3A6A 9937EC74 CEF36D97 F9106626 162F15E7

Trustpoint CA certificate accepted.

% Certificate successfully imported

Step 6: Import the SBC's Certificate

This step involves importing the digital certificate specifically issued for the SBC. This certificate is crucial for securing SIP TLS trunks and enabling encrypted communication. Copy and paste the SBC's PEM-formatted certificate data from your text editor into the terminal.

SBC-Example(config)#crypto pki import sip-trunk-cert certificate

Enter the base 64 encoded CA certificate.

End with a blank line or the word "quit" on a line by itself

-----BEGIN CERTIFICATE-----

LkNsYXNzIDMgUHVibGljIFByaW1hcnkgQ2VydGlmaWNhdGlvbiBBdXRob3JpdHkwHhcNOTcwNDE3MDAwMDAwWhcNMTExMDI0MjM5NTkZCBujEfMB0GA1UEChMWVmVy1RPuKSvD5HKNRO3RrCAJLeH24RkFOLA9D59/+J4C3IYChmFOJl9en5IeDCSk9dBwE88mw0M9SR2egi5SX7w+xmYpAY5Okiy8RnUDgqxz6dl+C2fvVFIa

-----END CERTIFICATE-----

quit

The output displayed here has been truncated for readability.

Upon successful import, the system will confirm:

% Router Certificate successfully imported

Step 7: Configure SIP User Agent (SIP-UA) for TLS Signaling

The SBC must be explicitly configured to utilize its newly imported certificate for all TLS signaling operations. This ensures that SIP communications are encrypted and authenticated using the established PKI infrastructure.

SBC-Example(config)#sip-ua

SBC-Example(config-sip-ua)#crypto signaling default trustpoint sip-trunk-cert strict cipher

# Call Routing Configuration

This document outlines the essential call routing configurations required to support SBC functionality, in compliance with UCR 2013, Change 2 guidelines. The configurations presented herein utilize the example network depicted in Figure 2 as a foundational template.

Figure 2: Call Routing Configuration Example

The central component of the diagram illustrates a redundant pair of Cisco devices, operating as a unified SBC. All ingress and egress communications to and from the SBC, on both network segments, are facilitated via VRRP IP addresses. On one side, the SBC interfaces with a single call control agent, which may be an LSC, MFSS, or WAN SS. Conversely, on the other side, the SBC interacts with potentially multiple other SBCs situated within the broader DISN core infrastructure.

Step 1: Disable Toll Fraud Prevention

Voice-enabled Cisco IOS routers incorporate a toll fraud prevention feature that is enabled by default. This parameter is not requisite for SBC operation and must be explicitly disabled using the following command sequence:

voice service voip

no ip address trusted authenticate

Upon entering the command, confirmation is required before the configuration is applied:

SBC-Example(conf-voi-serv)#no ip address trusted authenticate

The command DISABLES the ip address authentication on incoming

H.323 or SIP trunk calls for toll fraud prevention supports.

Continue? [confirm]y

Step 2: Configure the SBC for Unified Border Element Mode

To enable Unified Border Element (UBE) functionality, configure the SBC with the following command:

voice service voip

mode border-element

After applying this command, it is imperative to save the configuration and reload the SBC for the changes to take effect.

Step 3: Enable Global VoIP and SIP-UA Commands

These commands activate the necessary global configurations to satisfy the UCR guidelines. Unless otherwise specified, these commands must be configured precisely as shown.

voice service voip

rtcp keepalive

rtcp all-pass-through

rtcp-port range 16384 32766

media anti-trombone

allow-connections sip to sip

no supplementary-service sip moved-temporarily

no supplementary-service sip handle-replaces

modem passthrough protocol codec g711ulaw

h323

call service stop

sip

bind control source-interface GigabitEthernet0/0

bind media source-interface GigabitEthernet0/0

session transport tcp tls

session refresh

header-passing

error-passthru

referto-passing

asserted-id pai

asymmetric payload full

midcall-signaling passthru

pass-thru headers unsupp

pass-thru content unsupp

pass-thru content sdp mode non-rtp

sip-profile inbound

no call service stop

<configuration continued>

sip-ua

retry invite 3

retry rel1xx 1

timers trying 200

timers connection aging tls 4320

timers options 1000

transport tcp tlsv1.2

crypto signaling default trustpoint <Trustpoint Name> strict cipher

protocol mode dual-stack preference ipv4

no voice hunt unassigned-number

no voice hunt no-response

no voice hunt dest-out-of-order

no voice hunt invalid-number

no voice hunt b-cap-unauthorized

no voice hunt b-cap-unavail

no voice hunt b-cap-not-implemented

no voice hunt b-cap-restrict

no voice hunt no-voice-resources

The interface specified for bind control source-interface and bind media source-interface may require modification based on your specific network topology. This interface is critical for correct VRRP redundancy operation. This configuration is only applied if individual dial-peers (configured in a subsequent step) do not possess an explicit interface binding.

Step 4: Configure URI Classes

Call routing on an SBC is not solely dependent on Dialed Number Identification Service (DNIS). Inbound and outbound dial-peer selection is primarily determined by a combination of SIP route headers and the IP address embedded within the SIP URI. The following URI classes define the IP addresses of the call control agent and other peer SBCs with which this SBC will communicate.

Call Control Agent Class

The SBC is configured to point to a single Call Control Agent (CCA), which can be an LSC, MFSS, or WAN SS. This is a mandatory configuration. The IP address of the CCA must be defined under the "cca0" URI class:

voice class uri <cca0> sip

pattern <.*10.1.1.100>

Peer SBC Class

The SBC will interface with at least one other SBC. The IP addresses of these peer SBCs should be defined under distinct URI classes, such as SBC0, SBC1, SBC2, and so forth.

voice class uri <SBC0> sip

pattern <.*20.1.1.100>

Some lines in the pattern configuration end with a space before the closing quotation mark. This spacing is critical and must be precisely replicated as shown for correct functionality.

Step 5: Configure the SIP Profiles

The following SIP profiles perform essential SIP header normalization to meet the requirements for AS-SIP processing. These profiles should be configured exactly as presented, with the exception of highlighted areas that require updates specific to your environment. Copy-pasting the configuration is recommended to minimize errors.

voice class sip-copylist 1

sip-header SIP-Req-URI

sip-header Refer-To

sip-header Contact

voice class sip-profiles <102>

request INVITE peer-header sip SIP-Req-URI copy "(;rfrid=[0-9]+)@" u02

request INVITE peer-header sip Contact copy "(;[Cc][Cc][Aa]-[Ii][Dd]=[^;>]*)" u03

request ANY sip-header P-Asserted-Identity modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request INVITE sip-header To modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request INVITE sip-header SIP-Req-URI modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request INVITE sip-header SIP-Req-URI modify "@" "\u02@"

request INVITE sip-header From modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request ANY sip-header Contact modify ">" "\u03>"

request INVITE sip-header Route add "Route: <sip:iwbc01@20.1.1.100:5061;transport=tls;lr>"

request INVITE sdp-header Audio-Connection-Info modify "IP6  -" "IP6::"

voice class sip-profiles <724>

request INVITE peer-header sip SIP-Req-URI copy "(;rfrid=[0-9]+)@" u02

request INVITE peer-header sip Contact copy "(;[Cc][Cc][Aa]-[Ii][Dd]=[^;>]*)" u03

request ANY sip-header P-Asserted-Identity modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request INVITE sip-header To modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request INVITE sip-header SIP-Req-URI modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request INVITE sip-header SIP-Req-URI modify "@" "\u02@"

request INVITE sip-header From modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

<configuration continued>

request INVITE sip-header Route add "Route: <sip:iwbc01@10.1.1.100:5061;transport=tls;lr>"

request ANY sip-header Contact modify ">" "\u03>"

request ANY sip-header Allow-Header modify ", UPDATE" ""

request INVITE sdp-header Audio-Connection-Info modify "IP6  -" "IP6::"

Step 6: Configure the Dial-Peers

A unique dial-peer is necessary for each SIP URI class established in the preceding steps. Ensure that each dial-peer is configured with a unique dial-peer tag, appropriate destination and incoming URI classes, and an interface binding. The dial-peers for the example topology are provided below:

dial-peer voice <1724> voip

description <Interface to/from sa redcom slice 10.1.1.100>

preference 1

destination-pattern <312724...>.

modem passthrough protocol codec g711ulaw

session protocol sipv2

session target ipv4: <10.1.1.100:5061>

session transport tcp tls

incoming uri via <sardslice>

voice-class codec 1

voice-class sip resource priority mode passthrough

voice-class sip profiles <724>

voice-class sip options-keepalive up-interval 30 down-interval 10 retry 3

voice-class sip copy-list 1

voice-class sip bind control source-interface GigabitEthernet0/0/1

voice-class sip bind media source-interface GigabitEthernet0/0/1

dtmf-relay rtp-nte

srtp

ip udp checksum

dial-peer voice <1121> voip

description <Interface to/from to Cisco SS2 20.1.1.100>

preference 1

destination-pattern .T

modem passthrough protocol codec g711ulaw

session protocol sipv2

session target ipv4: <20.1.1.100:5061>

session transport tcp tls

incoming uri via <avayass1>

voice-class codec 1

<configuration continued>

voice-class sip resource priority mode passthrough

voice-class sip profiles <102>

voice-class sip options-keepalive up-interval 30 down-interval 10 retry 3

voice-class sip copy-list 1

voice-class sip bind control source-interface GigabitEthernet0/0/0

voice-class sip bind media source-interface GigabitEthernet0/0/0

dtmf-relay rtp-nte

srtp

ip udp checksum

Step 7: Enable Media Inactivity Detection

The Media Inactivity Timer feature allows the SBC to monitor Real-Time Protocol (RTP) packet flow and disconnect calls if no RTP packets are received within a configured timeframe. The SIP media inactivity timer specifically releases a session when RTP packets are not detected by the SBC after the specified interval. This feature is mandatory for high-availability deployments. Both devices within a redundant pair must be configured with identical timers.

ip rtcp report interval 3000

gateway

media-inactivity-criteria all

timer receive-rtp 86400

timer receive-rtcp 40

# SBC Configuration Example

This configuration assumes that all Information Assurance (IA) and Certificate configurations, as detailed in preceding sections, have been successfully implemented. The steps outlined below specifically address the configurations required for a Cisco SBC to provide combined functionality with a Cisco Session Controller.

Step 1: Disable Toll Fraud Prevention

Cisco IOS devices that are voice-enabled include a toll fraud prevention feature enabled by default. This feature is not required for SBC operation and must be disabled using the following command sequence:

voice service voip

no ip address trusted authenticate

Upon command entry, confirmation is required before the configuration is applied:

SBC-Example(conf-voi-serv)#no ip address trusted authenticate

The command DISABLES the ip address authentication on incoming

H.323 or SIP trunk calls for toll fraud prevention supports.

Continue? [confirm]y

Step 2: Configure the SBC for Unified Border Element Mode

To enable CUBE functionality, configure the SBC with the following command:

voice service voip

mode border-element

After applying this command, it is essential to save the configuration and reload the SBC for the changes to take effect.

Step 3: Enable Global VoIP and SIP-UA Commands

These commands activate the necessary global configurations to meet UCR guidelines. Unless otherwise specified, these commands must be configured precisely as shown.

voice service voip

rtcp keepalive

rtcp all-pass-through

rtcp-port range 16384 32766

media anti-trombone

media disable-detail-stats

allow-connections sip to sip

no supplementary-service sip moved-temporarily

no supplementary-service sip handle-replaces

modem passthrough protocol codec g711ulaw

h323

call service stop

sip

bind control source-interface GigabitEthernet0/0

bind media source-interface GigabitEthernet0/0

session transport tcp tls

session refresh

header-passing

error-passthru

referto-passing

asserted-id pai

asymmetric payload full

<configuration continued>

midcall-signaling passthru

pass-thru content unsupp

pass-thru headers unsupp

pass-thru content sdp mode non-rtp

sip-profile inbound

no call service stop

sip-ua

retry invite 3

retry rel1xx 1

timers trying 200

timers connection aging tls 4320

timers options 1000

transport tcp tlsv1.2

crypto signaling default trustpoint <Trustpoint Name> strict cipher

protocol mode dual-stack preference ipv4

no voice hunt unassigned-number

no voice hunt no-response

no voice hunt dest-out-of-order

no voice hunt invalid-number

no voice hunt b-cap-unauthorized

no voice hunt b-cap-unavail

no voice hunt b-cap-not-implemented

no voice hunt b-cap-restrict

no voice hunt no-voice-resources

The interface specified for bind control source-interface and bind media source-interface may require modification based on your specific network topology. This interface is crucial for correct Hot Standby Router Protocol (HSRP) redundancy operation. This configuration is only applied if individual dial-peers (configured in a subsequent step) do not possess an explicit interface binding.

Step 4: Configure URI Classes

Inbound and outbound dial-peer selection is primarily determined by the IP address embedded within the SIP URI. The following URI classes define the IP addresses of the CCAs and other peer SBCs with which this SBC may communicate.

Call Control Agent Class

An SBC can be configured to interact with multiple CCAs, which can include an LSC, the DoD MFSS, or a WAN SS. This is a mandatory configuration item. The IP addresses of the CCAs must be defined under distinct URI classes:

voice class uri <cca0> sip

pattern <.*10.1.1.100>

voice class uri <cca1> sip

pattern <.*10.1.1.101>

Peer SBC Class

The SBC will interact with at least one other SBC. The IP addresses of these peer SBCs should be defined under separate URI classes, such as SBC0, SBC1, SBC2, and so forth.

Step 5: Configure the SIP Profiles

The following SIP profiles execute the necessary SIP header normalization to meet the requirements for AS-SIP processing. These profiles should be configured exactly as shown. Copy-pasting the configuration is recommended after updating CCA-ID names and SIP IP addresses as required for your deployment.

Additionally, for Head-End Cisco Unified Border Element (HE-CUBE) deployments, the SIP profile requires modification to the SIP Reason header to ensure MLPP calls receive the appropriate Precedence and Preemption (PNT) tone.

All SBCs should include the sip-profile 100 below.

voice class sip-profiles <100>

request REFER peer-header sip Refer-To copy "<sip:(.*)>" u01

request INVITE peer-header sip SIP-Req-URI copy "(;rfrid=[0-9]+)@" u02

request ANY sip-header SIP-Req-URI modify "(;rfrid=[0-9]+)[^;@ ]" ""

request REFER sip-header Refer-To modify "sip:.*" "<sip:\u01>"

request INVITE sip-header To modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request INVITE sip-header SIP-Req-URI modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request INVITE sip-header SIP-Req-URI modify "@" "\u02@"

request ANY sdp-header Audio-Attribute modify "2^31" "2^20"

request ANY sdp-header Audio-Attribute modify "2^30" "2^20"

response ANY sdp-header Audio-Attribute modify "2^30" "2^20"

response ANY sdp-header Audio-Attribute modify "2^31" "2^20"

Example sip-profile 4 for Head End SBC – applicable only to HE SBCs.

voice class sip-profiles <4>

request INVITE peer-header sip SIP-Req-URI copy "(;rfrid=[0-9]+)@" u02

request ANY sip-header Contact modify ">" ";CCA-ID=ULXXXXX75>"

request INVITE sip-header Route add "Route: <sip:disasbc@201.2.1.4;lr>"

request ANY sip-header P-Asserted-Identity modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request INVITE sip-header To modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" @dsn.mil

<configuration continued>

request INVITE sip-header SIP-Req-URI modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request INVITE sip-header SIP-Req-URI modify "@" "\u02@"

request INVITE sdp-header Audio-Connection-Info modify "IP6  -" "IP6::"

Example sip-profile 3 for each Enterprise Site Controller (ESC) Site SBC.

voice class sip-profiles <3>

request REFER peer-header sip Refer-To copy "<sip:(.*)>" u01

request INVITE peer-header sip SIP-Req-URI copy "(;rfrid=[0-9]+)@" u02

request ANY sip-header P-Asserted-Identity modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request ANY sip-header Contact modify ">" ";CCA-ID=t2sbc>"

request INVITE sip-header Route add "Route: <t2sbc@201.2.1.36;lr>"

request ANY sip-header P-Asserted-Identity modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request REFER sip-header Refer-To modify "sip:.*" "<sip:\u01>"

request INVITE sip-header To modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request INVITE sip-header SIP-Req-URI modify "@[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+" "@dsn.mil"

request INVITE sip-header SIP-Req-URI modify "@" "\u02@"

request INVITE sdp-header Audio-Connection-Info modify "IP6  -" "IP6::"

Step 7: Configure the Dial-Peers

A unique dial-peer is required for each SIP URI class created in the previous steps. Ensure that each dial-peer is configured with a unique dial-peer tag, the respective destination and incoming URI classes, outbound proxy addresses, and interface binding. The dial-peers for the example topology are as follows:

dial-peer <voice 1> voip

description <Interface to CCA0 – 10.1.1.100>

preference 1

session protocol sipv2

session target ipv4:<10.1.1.100:5061>

session transport tcp tls

destination-pattern <314246....>

incoming uri via <cca0>

voice-class sip resource priority mode passthrough

voice-class sip profiles <100>

voice-class sip options-keepalive up-interval 120 down-interval 20 retry 3

voice-class sip bind control source-interface GigabitEthernet0/0

voice-class sip bind media source-interface GigabitEthernet0/0

dial-peer <voice 2> voip

description <Interface to CCA1 – 10.1.1.101>

preference 0

huntstop

session protocol sipv2

session target ipv4:<10.1.1.101:5061>

session transport tcp tls

destination-pattern <314246....>

incoming uri via <cca1>

voice-class sip resource priority mode passthrough

voice-class sip profiles <100>

voice-class sip options-keepalive up-interval 120 down-interval 20 retry 3

voice-class sip bind control source-interface GigabitEthernet0/0

voice-class sip bind media source-interface GigabitEthernet0/0

In a combined mode deployment, the SBC will route calls based on the destination pattern rather than solely on the URI. Therefore, it is highly recommended that a detailed dial plan be established prior to configuring these settings. In the example above, the destination pattern corresponds to the directly served Cisco Unified Communications Manager (CUCM).

dial-peer <voice 3> voip

description <Interface to Other SBC1 – 20.1.1.100>

session protocol sipv2

session target ipv4:<20.1.1.100:5061>

session transport tcp tls

destination-pattern .T

incoming uri via <SBC0>

voice-class sip resource priority mode passthrough

voice-class sip profiles <1>

voice-class sip options-keepalive up-interval 120 down-interval 20 retry 3

voice-class sip bind control source-interface GigabitEthernet0/1

voice-class sip bind media source-interface GigabitEthernet0/1

To minimize complexity, it is recommended that the destination pattern used in the second dial-peer (or subsequent dial-peers for external routing) be a "catch-all" pattern. This ensures that any call not resolved locally is forwarded to the external call control agent.

Step 8: Enable Media Inactivity Detection

The Media Inactivity Timer feature allows the SBC to monitor Real-Time Protocol (RTP) packet flow and disconnect calls if no RTP packets are received within the configured time period. The SIP media inactivity timer specifically releases a session when RTP packets for a call are not detected by the SBC after the specified interval. This mechanism guards against hung sessions that might result from a failover event where a normal call disconnect does not clear the call. This feature is mandatory when configuring high availability, and both devices within a redundant pair must be configured with identical timers.

ip rtcp report interval 3000

gateway

media-inactivity-criteria all

timer receive-rtp 86400

timer receive-rtcp 40

Sample Show Run for SBC

A sample “show run” from the Session Border Controller (SBC) can be found attached, filename “SBC_Show Run.txt”

# Related Documentation

The public configuration guide for the Cisco SBC is available at: cisco.com.

# List of Acronyms

| Acronym List |  |
| --- | --- |
| AAA | Authentication, Authorization, Accounting |
| CA | Certificate Authority |
| CAR | Cybersecurity Assessment Report |
| CCA | Call Control Agent |
| CUBE | Cisco Unified Border Element |
| DNIS | Dialed Number Identification Service |
| DoD | Department of Defense |
| DP | Defesne Programs |
| DTR | Desktop Review |
| ESR | Embedded Services Routers |
| FHRP | First Hop Redundancy Protocol |
| FQDN | Fully Qualified Domain Name |
| FTP | File Transfer Protocol |
| GCT | Global Certification Team |
| HA | High Availability |
| HTTP | Hypertext Transfer Protocol |
| HTTPS | Hypertext Transfer Protocol Secure |
| IA | Information Assurance |
| ICR | Internal Complainace Review |
| Ikev2 | Internet Key Exchange Version 2 |
| IP | Internet Protocol |
| IPsec | Internet Protocol Security |
| ISAkmp | Internet Security Ass. and Key Management Protocol |
| ISE | Identity Service Engine |
| JITC | Joint Interoperability Test Command |
| MFA | Multi-factor Authentication |
| MFSS | Multifunctional Softswitch |
| MUDG | Military Unique Deployment Guide |
| OCSP | Online Certificate Status Protocol |
| PEM | Privacy Enhance Mail |
| PKI | Public Key Infrastructure |
| SBC | Session Border Controller |
| SIP | Session Initiation Protocol |
| SSH | Secure Shell |

| Acronym List (continued) |  |
| --- | --- |
| STIG | Security Technical Implementation Guide |
| SUT | System Under Test |
| TACACS+ | Terminal Access Controller Access Control System Plus |
| TCP | Transmission Control Protcol |
| TFTP | Trivial File Transfer Protocol |
| TLS | Transport Layer Security |
| UBE | Unified Border Element |
| UCR | Unified Capabilities Requirements |
| URI | Uniform Resource Locator |
| VRRP | Virtual Router Redundancy Protocol |
| VTY | Virtual Terminal |
| WAN SS | Wide Area Network Softswitch |