# System Description

# Cisco Global Certification Team (GCT), Defense Programs (DP) Internal Compliance Review (ICR) SBC23 Certification Test

# This document presents a high-level overview of the system under evaluation, outlining its core functionalities and providing a concise discussion of its individual components and their respective applications. The purpose of this guide is to furnish readers with a foundational understanding of the system and its capabilities. For comprehensive technical details regarding the solution or any of its constituent components, readers are encouraged to consult Cisco’s official website at www.cisco.com.

| Version | Date | Change | Editor |
| --- | --- | --- | --- |
| 1.0 | Dec 2023 | Initial Version | GCT DP Collaboration |
| 2.0 | Sep 2024 | Update for DTR 1 | GCT DP Collaboration |

Table of Contents

System Description	2

Detailed Component Information	3

Desktop Review (DTR) 1	4

DTR Detailed Component Information	5

Management Description	6

# System Description

The Cisco Session Border Controller (SBC) enables secure and reliable IP-to-IP AS-SIP communications between Session Controllers (SCs) that support the UCR Alternative B failover mechanism and their associated end instruments, across the Unified Capabilities (UC) Network. The SBC interfaces with DISN UC systems via an AS-SIP interface, delivering robust IP-to-IP connectivity between edge components (Local Session Controller [LSC] / Enterprise Session Controller [ESC]) and core infrastructure (Softswitch) within the UC environment. Key functionalities of the SBC include AS-SIP signaling, comprehensive encryption services, and facilitation of end-to-end secure communications across the UC Network. Additionally, the solution can be integrated with a Cisco Session Controller to provide advanced interworking and SBC functionalities. The Cisco SBC solution is available on multiple hardware platforms, including the Cisco ISR 4000 Series, ASR 1000 Series, Catalyst 8000V, Catalyst 8200 Series, and Catalyst 8300 Series.

Component Description

The following components are included in the solution.

- Session Border Controller (SBC).  The SBC is an intelligent unified communications network border element that provides session border controller (SBC) functions that help enable end-to-end IP-based transport of voice, video, and data between independent unified communications networks and the UC network.

- Combined Session Border Controller (SBC) with Interworking Gateway (IWG).  The SBC combined with IWG type deployment provides critical SIP message normalization functionality that allows for interoperability between components within the Unified Capabilities architecture for the Cisco SC.  The combined SBC and IWG is an integrated Cisco IOS XE software application that runs on the Cisco ISR 4000 series, ASR 1000 series, Catalyst 8000V, Catalyst 8200 Series and Catalyst 8300 Series of hardware platforms.

Cisco Catalyst 8000V Edge Software Data Sheet

https://www.cisco.com/c/en/us/products/collateral/routers/catalyst-8000v-edge-software/datasheet-c78-744101.html?oid=dstetr023051

Cisco Catalyst 8200 Series Edge Platforms Data Sheet

https://www.cisco.com/c/en/us/products/collateral/routers/catalyst-8200-series-edge-platforms/nb-06-cat8200-series-edge-plat-ds-cte-en.html?oid=dstetr024667

Cisco Catalyst 8300 Series Edge Platforms Data Sheet

https://www.cisco.com/c/en/us/products/collateral/routers/catalyst-8300-series-edge-platforms/datasheet-c78-744088.html

Cisco 4000 Series Integrated Services Routers

http://www.cisco.com/c/en/us/products/collateral/routers/4000-series-integrated-services-routers-isr/datasheet-c78-732542.html

Cisco ASR 1000 Series Aggregation Services Routers Data Sheet Routers

http://www.cisco.com/c/en/us/products/collateral/routers/asr-1000-series-aggregation-services-routers/datasheet-c78-731632.html

# Detailed Component Information

| Component | Release | Sub-component | Function |
| --- | --- | --- | --- |
| Session Border Controller on 4321 ISR Session Border Controller on 4331 ISR Session Border Controller on 4351 ISR Session Border Controller on 4431 ISR Session Border Controller on 4451-X ISR Session Border Controller on 4461 ISR | IOS XE 17.12 | N/A | Session Border Controller provides connectivity to AS-SIP trunks. |
| Interworking Gateway/Session Border Controller on 4321 ISR Interworking Gateway/Session Border Controller on 4331 ISR Interworking Gateway/Session Border Controller on 4351 ISR Interworking Gateway/Session Border Controller on 4431 ISR Interworking Gateway/Session Border Controller on 4451-X ISR Interworking Gateway/Session Border Controller on 4461 ISR | IOS XE 17.12 | N/A | Interworking Gateway/Session Border Controller, provides connectivity to AS-SIP trunks. |
| Session Border Controller on ASR 1006 Session Border Controller on ASR 1006-X | IOS XE 17.12 | N/A | Session Border Controller provides connectivity to AS-SIP trunks. |
| Interworking Gateway/Session Border Controller on ASR 1006 Interworking Gateway/Session Border Controller on ASR 1006-X | IOS XE 17.12 | N/A | Interworking Gateway/Session Border Controller, provides connectivity to AS-SIP trunks. |
| Session Border Controller on C8300-1N1S-6T Session Border Controller on C8300-1N1S-4T2X Session Border Controller on C8300-2N2S-6T Session Border Controller on C8300-2N2S-4T2X | IOS XE 17.12 | N/A | Session Border Controller provides connectivity to AS-SIP trunks. |
| Interworking Gateway/Session Border Controller on C8300-1N1S-6T Interworking Gateway/Session Border Controller on C8300-1N1S-4T2X Interworking Gateway/Session Border Controller on C8300-2N2S-6T Interworking Gateway/Session Border Controller on C8300-2N2S-4T2X | IOS XE 17.12 | N/A | Interworking Gateway/Session Border Controller, provides connectivity to AS-SIP trunks. |
| Session Border Controller on C8200-1N-4T Session Border Controller on C8200L-1N-4T | IOS XE 17.12 | N/A | Session Border Controller provides connectivity to AS-SIP trunks. |
| Interworking Gateway/Session Border Controller on C8200-1N-4T Interworking Gateway/Session Border Controller on C8200L-1N-4T | IOS XE 17.12 | N/A | Interworking Gateway/Session Border Controller, provides connectivity to AS-SIP trunks. |
| Session Border Controller on C8000v | IOS XE 17.12 | N/A | Session Border Controller provides connectivity to AS-SIP trunks. |
| Interworking Gateway/Session Border Controller on C8000v | IOS XE 17.12 | N/A | Interworking Gateway/Session Border Controller, provides connectivity to AS-SIP trunks. |

# Desktop Review (DTR) 1

This DTR updates the Session Border Controller (SBC) IOS XE software on the ASR 1006-X, the C8300 series, the C8200 series, and the C8000v series of router platforms, on SBC. The IOS XE software version is being updated from 17.12 to 17.15.

Request certification through similarity based on SBC TN: 2104001, DTR62.

This DTR Clears POAM CIS-1824-00, SUT Has Miss-Match Audio Port Negotiation.

Release Notes for all devices will be provided once they become available, expected Sept 2024.

# DTR Detailed Component Information

| Component | Release | Sub-component | Function |
| --- | --- | --- | --- |
| Interworking Gateway on ASR 1006-X | IOS XE 17.15 | N/A | Interworking Gateway, provides connectivity to AS-SIP trunks |
| Session Border Controller on ASR 1006-X | IOS XE 17.15 | N/A | Session Border Controller provides connectivity to AS-SIP trunks. |
| Interworking Gateway/Session Border Controller on ASR 1006-X | IOS XE 17.15 | N/A | Interworking Gateway/Session Border Controller, provides connectivity to AS-SIP trunks. |
| Interworking Gateway on C8300-1N1S-6T1 Interworking Gateway on C8300-1N1S-4T2X Interworking Gateway on C8300-2N2S-6T1 Interworking Gateway on C8300-2N2S-4T2X1 | IOS XE 17.15 | N/A | Interworking Gateway, provides connectivity to AS-SIP trunks. |
| Session Border Controller on C8300-1N1S-6T1 Session Border Controller on C8300-1N1S-4T2X Session Border Controller on C8300-2N2S-6T1 Session Border Controller on C8300-2N2S-4T2X1 | IOS XE 17.15 | N/A | Session Border Controller provides connectivity to AS-SIP trunks. |
| Interworking Gateway/Session Border Controller on C8300-1N1S-6T1 Interworking Gateway/Session Border Controller on C8300-1N1S-4T2X Interworking Gateway/Session Border Controller on C8300-2N2S-6T1 Interworking Gateway/Session Border Controller on C8300-2N2S-4T2X1 | IOS XE 17.15 | N/A | Interworking Gateway/Session Border Controller, provides connectivity to AS-SIP trunks. |
| Interworking Gateway on C8200-1N-4T Interworking Gateway on C8200L-1N-4T2 | IOS XE 17.15 | N/A | Interworking Gateway, provides connectivity to AS-SIP trunks. |
| Session Border Controller on C8200-1N-4T Session Border Controller on C8200L-1N-4T2 | IOS XE 17.15 | N/A | Session Border Controller provides connectivity to AS-SIP trunks. |
| Interworking Gateway/Session Border Controller on C8200-1N-4T Interworking Gateway/Session Border Controller on C8200L-1N-4T2 | IOS XE 17.15 | N/A | Interworking Gateway/Session Border Controller, provides connectivity to AS-SIP trunks. |
| Interworking Gateway on C8000v | IOS XE 17.15 | N/A | Interworking Gateway, provides connectivity to AS-SIP trunks. |
| Session Border Controller on C8000v | IOS XE 17.15 | N/A | Session Border Controller provides connectivity to AS-SIP trunks. |
| Interworking Gateway/Session Border Controller on C8000v | IOS XE 17.15 | N/A | Interworking Gateway/Session Border Controller, provides connectivity to AS-SIP trunks. |
| Notes: - Certification through similarity based on the C8300-1N1S-4T2X - Certification through similarity based on the C8200-1N-4T |  |  |  |

# Management Description

The SBCs are managed via SSHv2 on the management network or via a console cable with a terminal connection. Security and auditing can be performed through configuration to a Cisco Identity Services Engine (ISE) server or similar Site-Owned Remote Auxiliary Equipment (RAE) services that are integrated with Active Directory and syslog server to collect alarm and modification information of the system.

Obtaining Further Documentation and Support

For information on obtaining documentation, obtaining support, providing documentation feedback, security guidelines, and recommended aliases and general Cisco documents, visit Cisco’s public website at:

http://www.cisco.com/